🧠 PROJECT 02 — INTER-VLAN ROUTING (ROUTER-ON-A-STICK)

📌 Scenario

A company has three departments connected to a single access switch:

- Finance Department

- IT Department

- HR Department

The departments are already separated using VLANs on the switch.
Management now requires communication between departments while maintaining logical separation.

You are asked to implement Inter-VLAN Routing using a single router interface.

📋 Network Requirements

VLAN Information

| VLAN ID | VLAN Name | Required Hosts |
| --- | --- | --- |
| 30 | Finance | 30 |
| 20 | IT | 20 |
| 10 | HR | 10 |

IP Addressing

- Use the 192.168.10.0/24 network.

- Apply VLSM to allocate subnets for each VLAN based on host requirements.

- Assign the first usable IP of each subnet to the router as the default gateway.

🧩 Devices to Use

- 1 × Cisco Router

- 1 × Cisco Switch

- 6 × End Devices (PCs / Laptops)

🔧 Tasks to Perform

1️⃣ IP Addressing

- Design the VLSM addressing scheme.

- Assign IP addresses and subnet masks to all end devices.

- Configure the default gateway on each PC.

2️⃣ Switch Configuration

- Ensure VLANs 10, 20, and 30 exist.

- Assign access ports to the appropriate VLANs.

- Configure one switch port as a trunk to connect to the router.

3️⃣ Router Configuration

- Configure subinterfaces on the router for each VLAN.

- Apply 802.1Q encapsulation.

- Assign IP addresses to each subinterface.

4️⃣ Verification

- Verify that devices in the same VLAN can communicate.

- Verify that devices in different VLANs can communicate.

- Use ping to confirm connectivity.

✅ Expected Outcome

- All VLANs remain logically separated.

- Inter-VLAN communication is successful through the router.

- The router acts as the default gateway for all VLANs.

📝 Deliverables (for GitHub)

- Packet Tracer file (.pkt)

- Screenshots of:

  - VLAN configuration

  - Trunk configuration

  - Router subinterfaces

  - Successful inter-VLAN pings

- README.md explaining the design and results

🔒 IMPORTANT RULE

❌ Do NOT use a Layer-3 switch
❌ Do NOT use multiple router interfaces

👉 Router-on-a-Stick ONLY